# LoongSocket LS-W1 S3 N16R8 API Reference

Generated from docs/API markdown files.

# README.md

# LoongSocket LS-W1 S3 N16R8 API

This folder documents the firmware API used by the

FW-LS-W1-S3-N16R8 project.

## Module

• Project: FW-LS-W1-S3-N16R8

• Module P/N: LS-W1-1119-38P-S3U-N16R8

• MCU: ESP32-S3

• Flash: 16 MB

• PSRAM: 8 MB

• ADC: MCP3208, 8 channels, 12 bit, 2500 mV reference

• IDF: 5.5.4

## Components

| Component | Header | Purpose |
| --- | --- | --- |
| BMS | ls_bms.h | Board identity, reset reason, memory and safe-mode status |
| USB | ls_usb.h | Native USB Serial/JTAG status and recovery |
| Storage | ls_storage.h | NVS-backed storage with write-on-change |
| MCP3208 | ls_mcp3208.h | External ADC acquisition, filtering, calibration and diagnostics |
| I/O | ls_io.h | GPIO, PWM and board reserved-pin protection |
| Board Test | ls_board_test.h | Production/demo helpers for PWM, square-wave and ADC PASS/NO PASS |

## Startup order

Recommended initialization order:

ls_bms_init();

The current Kickstarter monitor demo follows this order in main/main.cpp.

## Documentation Files

• LS_BMS_API.md

• LS_USB_API.md

• LS_STORAGE_API.md

• LS_MCP3208_API.md

• LS_IO_API.md

• LS_BOARD_TEST_API.md

• LS_API_STATUS.md

# LS_API_STATUS.md

# LS API Status

Status date: 2026-08-29

## Implemented

| Area | Status |
| --- | --- |
| BMS identity/status | Implemented |
| USB Serial/JTAG status | Implemented |
| USB recovery hook | Implemented, basic |
| NVS storage wrappers | Implemented |
| Write-on-change storage | Implemented |
| MCP3208 single read | Implemented |
| MCP3208 all-channel read | Implemented |
| MCP3208 burst read | Implemented |
| MCP3208 filter/calibration/range hooks | Implemented |
| MCP3208 hardware info | Implemented |
| MCP3208 diagnostics | Implemented |
| GPIO configure/read/write | Implemented |
| Reserved-pin protection | Implemented |
| Reserved-pin listing | Implemented |
| PWM start/set/stop | Implemented |
| PWM status query | Implemented |
| Board routing test helpers | Implemented |
| Kickstarter monitor demo | Implemented |

## Pending / Future

| Area | Notes |
| --- | --- |
| USB command console | Needed for production/service workflow |
| USB locked service commands | Needed before exposing dangerous operations |
| Boot/recovery procedure | Needs final policy for products without EN/BOOT access |
| Production report format | CSV/PDF or JSON report still pending |
| License/provisioning integration | Future factory workflow |
| Network/WiFi API | Not part of this first S3 API slice |
| Ethernet SPI option | Deferred |

## Current Validation

• Build validated with ESP-IDF 5.5.4.

• Monitor demo flashed and checked on COM8.

• S3 module detected as ESP32-S3 with 16 MB flash and 8 MB PSRAM.

• MCP3208 routing previously tested on CH0..CH7.

# LS_BMS_API.md

# LS BMS API

Header: components/ls_bms/include/ls_bms.h

The BMS component gives basic board-management information. It is the first API

to initialize because other services can report their status through it later.

## Main Types

ls_bms_status_t

• chip_model

• mac

• reset_reason

• free_heap_bytes

• minimum_free_heap_bytes

• psram_bytes

• boot_failure_count

• nvs_ready

• safe_mode_recommended

• usb_ready

• usb_connected

• usb_recovery_available

## Functions

esp_err_t ls_bms_init(void);

## Notes

• ls_bms_mark_healthy() should be called after the application has completed a

stable startup.

• safe_mode_recommended is intended for future protection/recovery logic.

• Current demo logs chip, MAC, reset reason, heap and PSRAM.

# LS_USB_API.md

# LS USB API

Header: components/ls_usb/include/ls_usb.h

The USB component tracks the native ESP32-S3 USB Serial/JTAG service and exposes

a small recovery API.

## Main Types

ls_usb_status_t

• initialized

• driver_owned

• connected

• init_failures

• recovery_attempts

• last_error

## Functions

esp_err_t ls_usb_init(void);

## Notes

• The S3 module has no easy external access to EN/BOOT in the final product, so

USB robustness is important.

• Keep USB commands explicit and safe. Future production commands should be

gated by service mode or password.

# LS_STORAGE_API.md

# LS Storage API

Header: components/ls_storage/include/ls_storage.h

The storage component wraps NVS and only commits values when their content

changes. This reduces unnecessary flash writes.

## Main Types

ls_storage_status_t

• initialized

• committed_writes

• skipped_writes

• last_error

## Functions

esp_err_t ls_storage_init(void);

## Notes

• Use short namespaces and keys.

• Use ls_storage_get_status() during endurance tests to see committed and

skipped writes.

# LS_MCP3208_API.md

# LS MCP3208 API

Header: components/ls_mcp3208/include/ls_mcp3208.h

This is the preferred public name for the external ADC API. Older ls_adc.h

symbols remain available for compatibility.

## Hardware Profile

• ADC: MCP3208

• Channels: 8

• Resolution: 12 bit

• Reference: 2500 mV

• SPI host: SPI2

• CS: GPIO3

• MOSI: GPIO11

• CLK: GPIO12

• MISO: GPIO13

## Main Types

The ls_mcp3208_* types are aliases of the compatibility ls_adc_* types:

• ls_mcp3208_filter_t

• ls_mcp3208_filter_config_t

• ls_mcp3208_calibration_t

• ls_mcp3208_range_t

• ls_mcp3208_sample_t

• ls_mcp3208_status_t

• ls_mcp3208_burst_t

• ls_mcp3208_hardware_info_t

## Main Functions

ls_mcp3208_init();

## Limits

• Channels: 0..7

• Burst samples: max LS_MCP3208_MAX_BURST_SAMPLES

• Raw counts: 0..4095

## Notes

• ls_mcp3208_read_burst() is used by the board-test component, so production

validation and application code share the same acquisition path.

• Direct PWM-to-ADC wiring validates routing, not precision calibration.

# LS_IO_API.md

# LS I/O API

Header: components/ls_io/include/ls_io.h

The I/O API controls GPIO and PWM while protecting board-reserved pins.

## Reserved Pins

The current board profile reserves:

| GPIO | Reason |
| --- | --- |
| 0 | boot strap |
| 3 | MCP3208 chip select |
| 11 | MCP3208 MOSI |
| 12 | MCP3208 clock |
| 13 | MCP3208 MISO |
| 19 | USB D- |
| 20 | USB D+ |

## Main Types

• ls_io_mode_t

• ls_io_pull_t

• ls_io_config_t

• ls_io_status_t

• ls_io_reserved_pin_t

• ls_io_pwm_status_t

## Functions

esp_err_t ls_io_init(void);

## PWM Limits

• PWM channels: LS_IO_MAX_PWM_CHANNELS

• Frequency: 10..20000 Hz

• Duty: 0..10000

## Notes

• PWM claims a pin until ls_io_pwm_stop() is called.

• ls_io_get_reservation() returns ESP_ERR_NOT_SUPPORTED for protected pins.

# LS_BOARD_TEST_API.md

# LS Board Test API

Header: components/ls_board_test/include/ls_board_test.h

The board-test API is a helper layer for production checks, laboratory routing

tests and the Kickstarter monitor demo.

## PWM Sources

The helper starts two PWM sources:

| Signal | GPIO | Frequency | Duty |
| --- | --- | --- | --- |
| PWM_CH1 | GPIO6 | 8 kHz | 38.00 % |
| PWM_CH2 | GPIO7 | 8 kHz | 76.00 % |

## Square-Wave Pins

The helper can output a 2 Hz square wave on user GPIOs. Reserved ADC SPI, native

USB and BOOT pins are excluded.

## Main Types

• ls_board_test_adc_config_t

• ls_board_test_adc_channel_t

• ls_board_test_adc_report_t

• ls_board_test_pin_t

## Functions

const ls_board_test_adc_config_t *ls_board_test_default_adc_config(void);

## PASS/NO PASS Meaning

• PASS | ready: no ADC input is connected, and no noise/saturation is detected.

• PASS | one channel detected: one ADC channel detects the PWM test signal.

• NO PASS: several channels are active/noisy/saturated or the ADC read failed.

This behavior is useful for demos because the board starts in a clean PASS state

even before a test wire is connected.